III.SOLUTIONS TO EXERCISES

The exercises that appear in the text are intended to stimulate student thinking, and promote a broader and deeper understanding of the subject matter.  Unfortunately, not every student will attempt to do these exercises as they read the text, and subsequently check their own analysis with the instructor.

I have successfully used many of these exercises in classroom, as a means of promoting student involvement and ensuring that the students understand the material.  At an appropriate time, I have posed one of these exercises to the class and asked them to discuss/solve it in small groups.  After a few minutes, I reconvene the class and call on one of the groups to present their analysis/solution.

Chapter 1:

Exercise 1-1:  The analytical system should be expanded to permit the measurement of more compounds.  Butene is an obvious example.  Desirably, the analyses should be extended to the point that all elemental balances can be closed.

Exercise 1-2:  The stoichiometric coefficients in Reaction (1-F) are one-half those in Reaction (1-E).  From Eqn.(1-18), r for Rxn.(1-F) must be twice that for Rxn.(1-E), i.e., r = 0.90.

Chapter 2:

Exercise 2-1:  Answer is given in book.

Exercise 2-2:  The use of N = 0 leads to a nonsensical answer, i.e., 1 = 3/2, 2 = 1, 3 = 0.  These individual orders are exactly the same as the orders for the forward reaction, so that the net rate does not depend of the concentration of any species.  This leads to a problem when the reaction comes to equilibrium.  Setting the forward and reverse rates equal leads to kf/kr = 1 = Keq.  Clearly, this is not the correct equilibrium expression.

Exercise 2-3:  This is an open-ended question, with few “wrong” answers.  Some important discussion items are:

- Phosgene is so toxic that it should not be stored or shipped.  The phosgene plant should be located adjacent to the facility that uses the phosgene, and the phosgene plant should be run “on-demand”, i.e., only when the COCl2-using facility is operating.

- The raw materials, CO and Cl2 also are toxic and hazardous.  Desirably, both should not be stored or shipped.  This is sometimes possible with CO, but is often not possible with Cl2.

- The phosgene plant should be designed so that leaks and releases, e.g., from a burst rupture disc or pressure relief valve, are contained.

- The plant should be alarmed so that non-essential personnel can be evacuated in the event of a release of phosgene, carbon monoxide, or chlorine.

Exercise 2-4:

a) When CA >> Km(T), -rA = Vmax.  At lower values of CA, -rA < vmax.

b) Km is the value of CA for which –rA = vmax/2, i.e., when CA = Km, -rA = vmax/2.  This may provide convenient way to measure Km.